ORVANTIA

Software Development | AI Solutions

1-9-34/G10, Street No.8, Habsiguda, Hyderabad, Telangana - 500007

Email: abhinaybakkera@orvantia.in   |   Website: https://orvantia.in   |   Phone: +91 8309403035

Udyam Registration No.: UDYAM-TS-31-0060097   |   PAN: GWRPB1132D

VPN Access Requirement for Remote Camera Connectivity

Proposal – AI-Based Attendance Tracking System

| To | Prayagh Consumer Care Pvt. Ltd. (Attn: IT / Network Team) |
| --- | --- |
| From | Orvantia |
| Project | AI-Based Attendance Tracking System |
| Date | 7 October 2026 |

1. Objective

To enable Orvantia to securely access the CP Plus camera/NVR remotely from the Orvantia machine for the development, testing, and operation of the AI-Based Attendance Tracking System.

Currently, the camera/NVR is accessible only when connected to the client’s 80-series network. To enable remote access, we propose establishing a VPN connection through the client’s Fortinet firewall.

2. Current Camera Configuration

| Parameter | Details |
| --- | --- |
| Device | CP Plus NVR |
| IP Address | 192.168.80.30 |
| Subnet Mask | 255.255.255.0 |
| Gateway | 192.168.80.1 |
| MAC Address | 5C:35:48:BF:15:2B |
| RTSP Port | 554* |

*To be verified during implementation.

3. Proposed Connectivity

CP Plus NVR

192.168.80.30

|

Factory Network

|

v

Fortinet Firewall

192.168.80.1

|

|  Encrypted VPN

v

Internet

|

v

Orvantia Remote Machine

Through the VPN, the Orvantia remote machine will securely access the NVR using its private IP address, without exposing the camera/NVR directly to the public internet.

4. Required from Client IT Team

The client’s IT/network team is requested to:

- Configure a suitable remote-access VPN on the existing Fortinet firewall.

- Create an authorized VPN account/configuration for Orvantia.

- Configure the required routing between the VPN and the 192.168.80.0/24 network.

- Allow the VPN user/system to access the NVR at 192.168.80.30.

- Allow the required RTSP service, expected on TCP port 554.

- Provide the VPN connection details securely to Orvantia.

- Assist with connectivity testing from the Orvantia remote machine.

Access should preferably be restricted to the required NVR and services rather than providing unrestricted access to the factory network.

5. Orvantia Responsibilities

After receiving the VPN configuration, Orvantia will:

- Configure the VPN on the designated remote machine.

- Establish and verify the VPN connection.

- Test connectivity to 192.168.80.30.

- Verify RTSP connectivity.

- Validate the camera stream.

- Integrate the camera stream with the AI-Based Attendance Tracking System.

6. Expected Result

Once configured, the Orvantia remote machine should be able to securely access the following endpoint through the VPN and use the camera stream for the Attendance Tracking System:

192.168.80.30:554

7. Next Step

We request the client’s IT team to confirm the Fortinet model and available VPN configuration and coordinate with Orvantia for the VPN setup and connectivity testing.

Thank you for your support. We look forward to your confirmation.

ORVANTIA

Software Engineering & AI Systems